Predictive Maintenance in the Railroad Industry: A Data-Driven Approach to Failure Forecasting

Student name

Student number

Institution

MGT480 – Capstone Project

Instructor:

Date:

# ABSTRACT

Predictive maintenance has emerged as a vital approach for promoting safety, reliability, and efficiency in contemporary rail transport. Conventional inspection techniques are ineffective in identifying early signs of structural faults in rails, which has introduced sensor-based monitoring as a vital innovation in rail transport. This capstone project explores a method for converting vibration and acoustic patterns into precise, data-driven models that anticipate early signs of defects in rail tracks using machine learning approaches coded in Python. This project utilizes the Railway Track Fault Detection Dataset, available on GitHub, which contains detailed data on vibration amplitudes, acoustic emissions, RMS energy, kurtosis, spectral entropy, FFT features, and categorized fault types. These multiple features are used to develop a quantitative research methodology for making precise predictions about early signs of faults, such as cracks, misalignments, and surface faults.

Numerous models, including Random Forest, Support Vector Machine, and XGBoost, are established and compared in this study using appropriate pre-processing, feature selection, cross-validation, and statistical significance tests. On applying Principal Component Analysis and feature importance, it has been found that RMS vibration energy, spectral entropy, kurtosis, and dominant FFT values are key factors for determining faults. The performance metrics of various models confirm good classification, and those of Random Forest are found to be superior. The analysis of sensor behaviour indicates prominent differences between normal and faulty paths in both time and frequency domains, where higher energy peaks, higher values of harmonic distortion, and random noise characterize faults.

These findings are consistent with other related literature, as a result of comparative analysis. This research has theoretical significance from a practical perspective, as it has demonstrated that open-source multimodal data and open-source Python-based tools provide value. The practical application of this analysis has demonstrated that railways can utilize predictive models for scheduling improvements, enhancing their digital twin models, and implementing early warning systems, ultimately leading to reduced delays and a decrease in disasters. Its limitations relate to sample representation, class imbalance, and limitations in signal processing granularity. In conclusion, this capstone project presents a robust and replicable model for interpreting data and predicting rail-track defects using sensor data, thereby further validating the significance of data analytics in enhancing rail safety.

Table of Contents

ABSTRACT	i

CHAPTER 1: INTRODUCTION	1

1.1 Predictive Maintenance in Railway Systems Background.	1

1.2 Disadvantages of Conventional Track Inspections.	1

1.3 Advances in Sensor-Based Monitoring Technologies	1

1.4 Role of Machine Learning in Early Fault Detection	2

1.5 Overview of the Railway Track Fault Detection Dataset	3

1.6 Problem Statement	4

1.7 Research Purpose	4

1.8 Research Questions	5

1.9 Significance of the Study	5

CHAPTER 2: LITERATURE REVIEW	7

2.1 Introduction	7

2.2 Predictive Maintenance in the Contemporary Rail Networks.	7

2.3 Machine-Learning Approaches to Track Fault Diagnosis	8

2.4 Sensor-Based Condition Monitoring (Vibration, Acoustic, Impact)	9

2.5 Feature Engineering and Signal-Processing Techniques.	9

2.6 Operational and Economic advantages of predictive maintenance	10

2.7 Gaps in Existing Research	11

2.8 Theoretical and Practical Contribution of This Study	12

2.9 Summary of the Literature Review	13

CHAPTER 3: DATASET AND RESEARCH DESIGN.	14

3.1 Introduction	14

3.2 Dataset Description	14

3.2.1 Source: GitHub Railway Track Fault Detection Dataset.	15

3.2.2 Sensor Modalities Captured	15

3.2.3 Type of Variables (Continuous, Binary, Categorical, Time-Series)	16

3.2.4 Labelled Fault Types and Class Distribution	17

3.3 Visual Model / Data Dictionary	17

3.4 Justification for Dataset Selection	18

3.5 Research Design	19

3.5.1 Quantitative Predictive-Analytics Orientation	19

3.5.2 Supervised Learning Framework	20

3.5.3 Alignment between Research Questions and Design	20

3.6 Ethical and licenses of open-source data.	21

CHAPTER 4: METHODOLOGY AND METHODS.	23

4.1 Introduction	23

4.2 Research Methodology	23

4.2.1 Quantitative Approach Justification	24

4.2.2 Predictive Modelling Orientation	25

4.2.3 Python in Data Analysis.	25

4.3 Methods	26

4.3.1 Data Cleaning and Pre-processing	26

4.3.2 Feature Engineering - RMS, Kurtosis, FFT, Spectral Entropy	27

4.3.3 Model Development - Random Forest, SVM, XGBoost	27

4.3.4 Model Evaluation Metrics (Accuracy, Precision, Recall, ROC-AUC)	28

4.3.5 Cross-Validation and Reproducibility	28

4.4 Statistical Hypothesis Testing Framework	29

4.4.1 Null and Alternative Hypotheses	29

4.4.2 Model Performance Significance Testing	29

4.5 Tools and Software Environment	30

4.5.1 Python Packages: Pandas, NumPy, SciPy, scikit-learn	30

4.5.2 Visualization Tools: Matplotlib, Seaborn	31

4.6 Limitations of the Methodology	32

4.7 Ethical Considerations	33

CHAPTER 5: RESULTS AND ANALYSIS	35

5.1 Introduction to Results	35

5.2 Model Performance Results	35

5.2.1 Performance of Classification	35

5.2.2 ROC-AUC Performance	36

5.2.3 Cross-Validation Consistency	36

5.2.4 Testing of Statistical Significance-ANOVA	37

5.3 Feature Importance Results	38

5.3.1 Random Forest Feature Ranking	38

5.3.2 Principal Component Analysis (PCA)	39

5.3.3 Diagnostic Insights from Sensor Features	40

5.4 Sensor-Signal Behaviour Analysis	41

5.4.1 Characteristics of Healthy and Faulty Waveforms	41

5.4.2 Frequency-Domain Behaviour from FFT	42

5.4.3 Detection of Abnormal Spikes, Odd Harmonics, and Energy Shifts	42

5.5 Summary of Key Analytical Findings	43

CHAPTER 6: DISCUSSION AND CONCLUSION	44

6.1 Introduction to Discussion	44

6.2 Discussion in Relation to Literature	44

6.3 Practical Implications for Railway Operations	45

6.4 Theoretical Contributions	46

6.5 Limitations of the Study	47

6.6 Recommendations for Future Research	48

6.7 Conclusion	49

REFERENCES	51

LIST OF FIGURES

Figure 1: Vibration signals	36

Figure 2: ROC Curves	37

Figure 3: Confusion Matrix, Random Forest	38

Figure 4: Feature Importance, Random Forest	39

Figure 5: PCA Scatter Plot	40

# CHAPTER 1: INTRODUCTION

## 1.1 Predictive Maintenance in Railway Systems Background.

With the growth of the railway networks and the increase in the importance of service reliability, predictive maintenance has proved to be the strategic priority of the railway operator. Instead of just using scheduled or reactive interventions, predictive maintenance uses sensor-obtained information and analysis to predict equipment degradation and suggest maintenance steps in advance of faults turning into service failures or safety accidents. Predictive approaches have been adopted in the railway industry because of the development of sensing, data storage, and machine learning, which combined make it possible to monitor the health of the tracks and rolling stock continuously and translate the unprocessed sensor data into an actionable indicator to plan maintenance (Aremu, 2020). The datasets that are publicly accessible and capture vibration, acoustic, and impact-force signals, like the Railway Track Fault Detection Dataset utilized in this capstone, allow reproducible research and serve as the empirical foundation upon which the fault-forecasting models, which can be applied in the operation of the real system, are built (GitHub Repository, n.d.; Le Nguyen, 2023).

## 1.2 Disadvantages of Conventional Track Inspections.

The old paradigm of track-inspection, as seen by manual walk-throughs, scheduled visual inspections, and constant replenishment of components, is becoming less and less effective in the modern rail system. Such methods frequently do not reveal the defects that are in the early stages and are embedded in rail geometry, ballast, or fastening systems, and are likely to be affected by under-detection of weak components, and prevent unnecessary maintenance when the elements are changed too soon (Xie et al., 2020). Practically, being dependent on planned maintenance may result in poor resource allocation and increased lifecycle expenses since not all interventions are in line with the actual state of assets. Moreover, manual checks are manual, time-consuming, and may be unsafe in some of the track setups. It states that sensor-based tracking and analytics (based on data), such as detecting the precursors of the failure that cannot be seen with the naked eye, can significantly minimize them (Binder et al., 2023; Ejaz & Bakhsh, 2023).

## 1.3 Advances in Sensor-Based Monitoring Technologies

The latest innovations in the sensing hardware and signal-processing software have upgraded the quality and resolution of data that can be used in monitoring the conditions of a rail. Continuous and high-frequency time-series data describing the dynamic interactions between rolling stock and track infrastructure are currently being generated by high-sensitivity accelerometers, acoustic microphones, wheelrail contact sensors, and on-board telemetry systems. Time- and frequency-domain features, including root-mean-square (RMS) energy, spectral entropy, kurtosis, and FFT-based coefficients, can be extracted using these multivariate data streams and have been demonstrated to be related to various modes of failure like cracks, misalignment, wear on the surface, and so on (Łuczak, 2024; Shafique et al., 2021). Empirical experiments also show that concurrent use of sensor modalities, such as the combination of vibration and acoustic channels, etc., leads to a stronger detection resolution due to the fact that various sensors record complementary physical ones of degradation (Prakash & Tadepalli, 2024; Lee, Kim, and Park, 2023). These technological advances, coupled with available, easily marked datasets on sites like GitHub, render the creation of reproducible, Python-based predictive maintenance analytical pipelines possible to be verified across numerous studies and operating situations (Aremu, 2020; GitHub Repository, n.d.).

## 1.4 Role of Machine Learning in Early Fault Detection

Machine learning has taken center stage in current predictive-maintenance strategies due to its capability of learning trends in big, complicated sensor data collections and providing dependable predictions of infrastructure deterioration. There are latent markers of cracks, misalignments, and surface defects in the vibration and acoustic signals of a railway operation that are not easily identified by a standard inspection process or using a threshold-based monitoring regime. Classification algorithms supervised learning algorithms (Support Vector Machines, Random Forests, and Gradient Boosting) have shown exemplary performance in classifying these types of fault when trained on well-labelled data (Binder et al., 2023). Non-linear interactions between time-series features of real mechanical behaviour of rail structures under dynamic loading are also extractable using machine learning to detect faults earlier and more accurately than manually interpretable or classical statistical approaches to the data (Aremu, 2020; Xie et al., 2020).

Machine learning is more than detecting; it is also about prediction, where the machines predict the possibility of a failure in the future that has been predicted by the sensor patterns of the past. This change from detection-focused maintenance to forecast-oriented maintenance provides the railway organization with the chance to respond to the faults before they develop into safety risks or service failures. The recent adoptions of machine-learning models in the academic literature and industrial settings demonstrate that the models are more efficient than fixed-interval maintenance in images of degradation and finding them earlier and more accurately, leading to the high reliability and lower cost of operation (Ejaz & Bakhsh, 2023; Aravind, Shah, and Surabhi, 2022). In the current work, machine learning will be the analytical core of the predictive-maintenance system, which will combine the vibration, acoustic, and frequency-domain characteristics into the classification and forecasting models created in Python.

## 1.5 Overview of the Railway Track Fault Detection Dataset

The data sample chosen to be used in this capstone project, which is the Railway Track Fault Detection Dataset that is publicly available on GitHub, provides an excellent starting point in the form of input to predictive-maintenance modelling. It includes raw and processed vibration and acoustic measurements taken on the pieces of the railway tracks in the operational conditions (GitHub Repository, n.d.). These measurements are able to record the dynamical interaction between the wheel-rail surfaces, and hence the extraction of time-domain signals like RMS energy, peak acceleration, and frequency-domain signals like spectral entropy and FFT magnitudes. Labelled fault categories are also given in the data set, and supervised machine-learning models can be trained to distinguish between healthy tracks, cracks, misalignments, and other structural defects.

The main strength of this data is its realism. Since the signals are not based on simulated laboratory measurements but on physical inspection measurements, they represent environmental noise, operational variability, and irregularities that define real-world railway dynamics. This enhances the ecological soundness of the predictive models and ensures that the insights extracted with the help of Python-based analysis can be applied to the industry. The option to use both types of features enables the research to assess a range of modelling methods and determine which features provide the most predictive power, an unanswered question in the existing literature (Le Nguyen, 2023; Xie et al., 2020). The results can also be rewritten, which is further promoted by the open-access nature of the dataset that allows other researchers to repeat or build on the results.

## 1.6 Problem Statement

In spite of the increased sensor implementations in contemporary railway systems, a significant number of operators continue to face difficulties when converting crude vibration and acoustical information to practical predictive-maintenance information. Manual and scheduled inspections simply do not see minor degradation in the early stages. In contrast, threshold-based control mechanisms are not always able to detect non-linear dynamics that are invisible before structural damage. Because of that, undetected defects and ineffective distribution of maintenance keep disrupting rail networks, endangering safety, and adding to their cost implications (Binder et al., 2023; Xie et al., 2020).

The main issue that this study aims to resolve is the fact that there are no solid analytical models that would be based on data and could convert multi-sensor railway signals into dependable failure anticipations. The railway operators have increased amounts of sensor data, but do not always have machine-learning models to analyse these signals and predict decay. Current literature indicates the possibility of advanced analytics, but not many use the publicly accessible datasets and explore the enhancement of predictive accuracy when using multimodal sensing (vibration and acoustic) to detect faults (Aremu, 2020; Le Nguyen, 2023). Thus, the proposed project will seal this gap by creating and testing Python-based machine-learning models at the Railway Track Fault Detection Dataset, a free, open-source dataset, in order to discover what sensor-derived features could be utilized to predict track faults precisely and consistently.

## 1.7 Research Purpose

This study aims at designing and testing a predictive-maintenance framework that is based on data and can forecast and detect the faults of the railway track based on high-resolution data from vibration and acoustic sensors. Every day, rail systems produce enormous torrents of sensor data, but most organizations cannot analytically process such data and transform it into prompt maintenance responses. Through supervised machine-learning analysis of the open-source Railway Track Fault Detection Dataset, this paper will show how analytics using Python can be used to ensure that railway operators are able to identify early signs of degradation, the type of faults, and how to predict failures before they turn into a safety or operational issue. The study aims at not only developing predictive models but also the ability to learn the features that have the most significant predictive potential, those that are RMS energy, spectral entropy, peak acceleration, acoustic signatures, or frequency-domain coefficients. By doing it, the study adds empirical data to the overall change in the paradigm of reactive or scheduled maintenance practices to the practice of data-driven asset management (Aremu, 2020; Binder et al., 2023; Xie et al., 2020).

## 1.8 Research Questions

The study is informed by three key research questions based on the gaps that have been discovered in the literature and the prevailing industry practice.

RQ1: Are data from vibration and acoustic sensors effective in predicting the existence of faults on railway tracks prior to their effects as failure of operations?

RQ2: What sensor signal-derived features (RMS energy, spectral entropy and peak acceleration) are the strongest predictors of track faults?

RQ3: Does the use of supervised machine-learning models, including those of Random Forest, Support Vector Machine, and XGBoost, yield better predictive accuracy than the conventional threshold-based fault detection technique?

## 1.9 Significance of the Study

The study has important operational, safety, and academic implications for the rail industry. In operational terms, the creation of correct predictive models makes it possible to plan more efficient maintenance, minimize downtime, and decrease the necessary costs of unnecessary repairs. Anticipating faults prior to their occurrence enables rail organizations to plan their resources more effectively and prevent abrupt interruption of their services. Another importance of the study is to improve the safety of passengers and reliability of assets since the defects are detected at an early stage and are easily missed by traditional inspections. By so doing, it goes hand in hand with the efforts of the global community on building resilience of essential transportation infrastructure.

Research-wise, the study adds to the literature on data-driven railway maintenance because of the publicly available, reproducible dataset. Numerous previous studies use proprietary data, which restricts comparability and transparency. This project enhances methodological openness by using the Railway Track Fault Detection Dataset on GitHub that facilitates the replication of research in the future (GitHub Repository, n.d.). In addition, the article contributes to the debate on multimodal sensing and feature relevance by comparing the effectiveness of the vibration and acoustic signals when used together in predictive behavior. This addition is essential since recent literature recommends further analysis of the relationship between the characteristics and the real-world fault mechanisms (Aremu, 2020; Le Nguyen, 2023; Xie et al., 2020). Generally, the results of this study can be used in the future to direct research and policymaking, and to assist the railway operators who aim to refresh their maintenance systems using machine-learning technologies.

# CHAPTER 2: LITERATURE REVIEW

## 2.1 Introduction

The swift development of predictive maintenance technologies in the railway sector has been influenced by the lightning of advances in sensing technologies, computing techniques, as well as the growing access to high-resolution operational information. Traditional maintenance techniques that are grounded on the scheduled maintenance or the reactive interventions face difficulties in offering the early signals of track wear, hence resulting in preventable safety hazards, delays, and losses in operations. The latest research points to the fact that introducing machine learning to maintenance processes helps rail operators to identify degradation indicators in their early stages and prevent the development of faults (Binder et al., 2023; Xie et al., 2020).

## 2.2 Predictive Maintenance in the Contemporary Rail Networks.

Predictive maintenance has become an inevitable phenomenon as rail systems become more complicated and require more and more efficiency and reliability. Research indicates that conventional inspection procedures tend to miss defects in structure at the early stage, especially when they are under the surface or in their complex fastening elements (Xie et al., 2020). Although manual inspections are still widespread, they are subjective, labour-intensive, and cannot offer real-time monitoring. This poses important knowledge lapses in assets that lead to derailments, service interruptions, and expensive emergency repairs.

The development of digital instrumentation and real-time data collection has enabled the move to predictive maintenance. As Aremu (2020) points out, the current railway infrastructure produces large volumes of sensor data, but to do something meaningful, there is a need to represent them with structure and provide the feature engineering that requires machine learning. In a similar regard, Binder et al. (2023) in their systematic review can prove that predictive models are more productive compared to traditional threshold-based systems, as they can detect cracks, misalignment, and geometry distortions earlier and more precisely.

It has also been found that the inclusion of predictive analytics in the decision-making process of operations lowers the lifecycle expenses and increases the reliability of the network. Investigating self-governing transit networks, Ejaz and Bakhsh (2023) have discovered that predictive maintenance, which uses artificial intelligence, can reduce the number of unexpected downtimes to a minimum, enhancing the continuity of service. Even though they are focused on autonomous transit systems, the principles of analysis, sensor-based anomaly detection, and supervised learning directly apply to the railway domain. The aggregate evidence supports the idea that predictive maintenance is not a concept in the experimental stage but rather an industry priority that is backed with solid empirical evidence.

## 2.3 Machine-Learning Approaches to Track Fault Diagnosis

Machine learning is essential in converting raw sensor information into applicable diagnostics of the railway track infrastructure. An increasing number of studies are able to prove the efficiency of the supervised learning models to identify and categorize rail defects. As an example, Binder et al. (2023) note Support Vector Machines, Random Forests, and Gradient Boosting as some of the most common top-performing algorithms in tracking the condition of tracks. Such models are so superior since they are able to handle non-linear trends of vibration and acoustic data trends that can generally indicate the emergence of cracks, corrugation, or misalignment.

Feature engineering and signal processing are also identified as important in the literature. Xie et al. (2020) demonstrate that characteristics based on frequency-domain transformations, including spectral entropy and FFT coefficients, are pretty effective in improving the accuracy of classification by localizing delicate mechanical vibrations that are not observable in time-domain signals. In a doctoral research on the topic of online machine-learning frameworks, Le Nguyen (2023) also shows that it is possible to deploy the models in real-time, and incremental learning approaches allow the models to adapt to new sensor data continuously.

Furthermore, Aravind, Shah, and Surabhi (2022) also provide examples of case studies to demonstrate that vehicle-based vibration analytics can efficiently respond to roadway and rail anomalies. Their results confirm the possibilities of supervised learning with the help of organized data like the one that was employed in this capstone. The Railway Track Fault Detection Dataset can be used to test such models, as it has the required diversity of vibration, acoustic, and feature-engineered inputs. All these works confirm the methodological decisions particularly the focus on Python-based supervised learning and multimodal sensor analysis.

## 2.4 Sensor-Based Condition Monitoring (Vibration, Acoustic, Impact)

The use of sensor-based monitoring devices has become the basis of present-day predictive-maintenance systems in the railway setting. Since visual inspection is not as practical as desired to detect defects in the subsurface and conceptualize the structural fatigue at an early stage, sensor instrumentation delivers a continuous flow of objective condition data. Vibration sensors usually are accelerometers on inspection trolleys or rolling stocks and capture the dynamic rail responses to wheel loads and have been shown to be very effective in alerting to anomalies related to cracks, joint loosening, or ballast degradation (Xie et al., 2020). These vibration signatures enable the analysts to follow the mechanical behaviour of the track in service, which provides subtle indicators of degradation even before it becomes apparent.

Acoustic sensing is an addition to the technology of vibration monitoring, with the difference that acoustic emissions are recorded at the wheel-rail interface. Frictional irregularities, surface wear, or defects that affect contact mechanics are also typical in changes in acoustic frequency and amplitude. It has been demonstrated that such acoustic patterns, combined with vibration data, can play a significant role in improving fault-detection performance because they are sensitive to various types of structural distress (Binder et al., 2023). Impact-force measurements also enhance the monitoring. These sensors measure anomalous forces due to rail discontinuities, deformation, or misalignment, which is a clear indication of abrupt structural perturbations.

Multimodal fault diagnosis. Utilizing vibration, acoustic, and impact sensors, e.g., those included in the Railway Track Fault Detection Dataset, has proven to be consistently beneficial. This is empowered by their combination to provide all-inclusive measurements of track health through the measurement of steady-state and transient behaviours, enhancing the accuracy and reliability of machine-learning-based prediction models (Aremu, 2020; Ravi Aravind et al., 2022).

## 2.5 Feature Engineering and Signal-Processing Techniques.

Deliberation of predictive-maintenance models is, therefore, very sensitive to the quality of the feature extraction and pre-processing used on the raw sensor signals. Raw vibration or acoustic records are rich in information, although the best use of this information is not achieved until signal-processing methods are used to extract structural patterns. Aremu (2020) has highlighted that it is necessary to convert raw sensor data into representations that can be trained with machine learning models to allow them to generalize and identify signs of degradation at an early stage.

Root-mean-square (RMS) energy, kurtosis, peak acceleration, and statistical variance are time-domain characteristics that are used to give information about the strength and the fluctuation of rail vibrations. These characteristics bring to the fore abrupt variations in amplitude, which tend to be associated with cracks or discontinuities. Periodic behaviours and harmonic distortions, which are manifested by frequency-domain features obtained with Fast Fourier Transform (FFT) and spectral analysis, signal misalignment or corrugation (Xie et al., 2020). Spectral entropy, predominant frequencies, and band-energy distributions give the classification models a chance to establish the categories of the fault with even greater confidence.

The development of signal-processing that Le Nguyen (2023) describes proves the possibility of deploying online machine-learning systems, which are based on real-time feature extraction and model refinement. These approaches combine time- and frequency-domain information and therefore guarantee the predictive models are sensitive to changing operational circumstances. It has also been empirically confirmed that combined engineering of multimodal features leads to an increase in accuracy in the case of vibration and acoustic features, since each sensor channel encodes different mechanical properties (Ravi Aravind et al., 2022).

## 2.6 Operational and Economic advantages of predictive maintenance

Predictive maintenance presents significant operational and cost benefits to the railway companies aiming to reduce disruptions, improve safety, and overcome lifecycle costs. The literature has consistently indicated that the switch to data-based strategies, as opposed to reactive or scheduled maintenance, can help to minimize unexpected failures, minimize downtimes, and optimize the use of labour. Ejaz and Bakhsh (2023) found that AI-controllable predictive models in autonomous transit networks allowed the mitigation of unscheduled maintenance interventions and enhanced the reliability of the system, which is directly transferred to traditional rail environments. They present the results of their findings that predictive analytics can be used not only to identify faults early but also to assist operators in prioritizing interventions based on the risk that is measured, instead of arbitrary timeframes.

Binder et al. (2023) also underscore the fact that predictive-maintenance implementation provides a reduction in inspection recovery and increased asset efficiency through the ability to perform specific repairs. Rather than making early replacements, the operators will be able to have maintenance scheduled according to the real status of the assets, hence reducing the costs of early replacement. Essentially, infrastructure protection-wise, the early identification of cracks, misalignment, or surface deformation would stop the progression into significant failures, causing prolonged closures and costly structural work.

It is also demonstrated in the operational studies that the predictive-maintenance structures enhance passenger safety by reducing derailment risks and maintaining the integrity of the tracks when operating in growing load and speed conditions (Xie et al., 2020). Predictive models using sensor-based monitoring have an important economic effect, which is to dramatically increase the life cycle of track components, minimise emergency-response costs, and simplify their resource planning. These recorded advantages support the relevance of the current research and its aim of creating predictive models that can be replicated through Python usage backed by actual multimodal sensor data in GitHub. The research will assist the railway operators in transforming their maintenance culture by improving the quality and legibility of fault predictions and guaranteeing their infrastructure resilience in the long term.

## 2.7 Gaps in Existing Research

Although the conventional prediction, maintenance, and evidence-based diagnostics have advanced significantly, there are still a number of gaps in existing literature on the topic of railway maintenance. One of the limitations that are common to research is the use of proprietary or inaccessible datasets, which complicates replication and comparative analysis of machine-learning systems (Aremu, 2020; Binder et al., 2023). Various investigators come up with highly specialized models that rely on internal railway data, which is not available to the general public. Thus, scholars and practitioners cannot legitimize algorithms or compare results. This decreases the external validity of results and retards the wider uptake of predictive maintenance in the industry.

The second gap has to do with the lack of research on the topic of multimodal sensing. Whereas vibration, acoustic, and impact-force sensors have been studied separately, there is a lack of literature that integrates these modalities into one unified machine-learning framework. As Xie et al. (2020) note, multimodal data will give complementary information on rail degradation, but limited research effort examines a systematic study of the impact of both vibration and acoustic signals on detecting faults. On the same note, feature-importance analysis, which finds out why some features are predictive, is not well-developed. Although a significant number of studies have reported high accuracy rates, they typically do not discuss much of what features (RMS, spectral entropy, FFT peaks, etc.) motivate the model performance (Le Nguyen, 2023). These mechanisms are essential to interpretable and reliable machine-learning systems that will be applicable in the real world.

Lastly, the performance of machine learning is well-documented; however, less literature is available on how predictive systems may be incorporated into operational processes. Strong potential can be traced in terms of cost savings and reliability improvement; however, there are gaps in terms of implementation, data governance, and real-time deployment (Ejaz & Bakhsh, 2023). The restrictions are an indicator of the necessity of an open, reproducible, multimodal, and operationally based research, which is exactly what this capstone project fulfills with its open dataset, sensor-based method, and Python-based modelling pipeline.

## 2.8 Theoretical and Practical Contribution of This Study

On the one hand, the theoretical framework of the study contributes to the development of the concept of utilizing multimodal sensor data, namely, vibration, acoustic, and impact-force signals, to develop predictive models that may help to detect faults in the track in the first place. The study makes contributions to reproducible machine-learning research in the railway field by showcasing an open-source dataset on GitHub, which arguably constitutes one of the most perennial literature gaps. Moreover, the paper compares the significance of time-domain and frequency-domain characteristics, which provide a more profound understanding of the features of signals that are associated with various forms of rail degradation. This level of understanding features reinforces theoretical understanding on the mechanistic track-deterioration and improves interpretability of machine-learning classifiers, which is essential to high-stakes transportation systems (Xie et al., 2020; Aremu, 2020).

In practice, the research indicates how the raw sensor data of the railways can be transformed into practical maintenance information through Python-based analytical systems by using libraries like NumPy, SciPy, pandas, and scikit-learn. It is in line with industry trends of data-driven asset management and can be used to assist railway organizations that want to minimize faults, optimize the maintenance regimes, and enhance safety performance (Binder et al., 2023; Ejaz & Bakhsh, 2023). The multimodal approach also provides operators with a much deeper look at the health of tracks and can probably identify faults earlier than vibration-based or acoustic-based systems. The research is based on the actual validation of these techniques on real-world data, which serves as a blueprint that can be applied to the railway agencies that want to implement predictive-maintenance analytics without using proprietary data and without intricate closed-source solutions.

## 2.9 Summary of the Literature Review

The literature regularly proves that predictive maintenance with the assistance of sensor technologies and machine-learning analytics comes with significant benefits in comparison with the conventional approaches to inspections. Vibration, acoustic, and impact sensors record significant patterns of the structure, which can be associated with initial deterioration, and supervised learning models, including Random Forests and SVMs, have demonstrated a high level of effectiveness in detecting and predicting faults (Xie et al., 2020; Binder et al., 2023). Research also indicates that predictive maintenance maximizes safety and minimizes costs and operational reliability in contemporary rail systems (Ejaz & Bakhsh, 2023).

Nevertheless, some gaps remain, especially in terms of limited access to open datasets, a lack of multimodal modelling, and an inability to explain features at all. Such gaps are the reason why the current research is necessary, as it uses Python-based machine-learning algorithms on an open-source Railway Track Fault Detection Dataset on GitHub. The research study has a theoretical and practical contribution to the development of intelligent rail-maintenance systems because it focuses on multimodal sensing and feature importance. The information provided in the literature is the basis of the research design and methodology in the next chapter.

# CHAPTER 3: DATASET AND RESEARCH DESIGN.

## 3.1 Introduction

This chapter provides the data and research design that informs the research framework of analysis. The capstone project, is based on a quantitative machine-learning-intensive method to predict faults in railway tracks based on the high-resolution vibration and acoustic measurements. The general goal of the research design is to match the problem statement of the challenge faced by rail operators in translating sensor data into viable maintenance information, with a methodology route that can fill that void. The empirical base of the study is the Railway Track Fault Detection Dataset that is available on GitHub and offers realistic and labelled sensor data that is appropriate to use in supervised learning experiments implemented using Python. In this chapter, thus, the data set, data structure, variables, and relevance are described, followed by the description of how the data would be used in the research purpose and the information it provides to the methodological decisions later.

## 3.2 Dataset Description

The data set in this paper will be that of the Railway Track Fault Detection Dataset, which is a publicly available, multimodal dataset located on GitHub. The dataset includes actual data on vibration and acoustic sensor measurements on the segments of railway tracks during the inspection checks. These measurements were made with accelerometers and acoustic microphones on movable inspection platforms and could measure physical contact between the wheel and the rail during the operational conditions.

The raw sensor streams are also included in the dataset, and the engineered features that are computed, which are root-mean-square (RMS) energy, peak acceleration, kurtosis, spectral entropy, and FFT-derived coefficients. Expansions of these properties in previous studies have been found to be highly associated with structural defects, including the presence of cracks, misalignment, and surface flaws (Aremu, 2020; Xie et al., 2020). Notably, the labelled results are included in the dataset, which shows whether each segment is healthy or defective, which makes it possible to use the supervised-learning structure.

The dataset was discussed to possess the following features to be used in more advanced analysis: high-resolution signals, multimodal sensing, continuous variables, categorical identifiers, and binary fault labelling. Both classification modelling and exploratory feature-importance analysis can be supported using these properties, and therefore, the dataset is a suitable and strong base for predictive maintenance research.

### 3.2.1 Source: GitHub Railway Track Fault Detection Dataset.

The data can be found in the open GitHub repository, named Railway Track Fault Detection Dataset. The dataset URL is: https://github.com/durgeshsahu16/Railway-Track-fault-Detection-Project

The reason why this open-source dataset was selected is that it matches the research problem: predictive-maintenance models are often unavailable in standard, transparent datasets because the amount of sensor data collected by railway organizations is significant. GitHub data does not just give raw sensor data, but contains feature-engineered variables that allow the experimentation of machine learners. Its open-access nature promotes reproducibility, transparency, and additional validation, which responds to the research gap brought out in the literature on over-reliance on proprietary datasets (Binder et al., 2023; Le Nguyen, 2023).

Also, the GitHub data can be used to test the multimodal modelling strategy. Owing to the inclusion of vibration and acoustic channels, researchers can examine the effects of modalities combining to determine their impact on detection accuracy, which is not a well-studied area in current literature. The well-organized format of the dataset, labelled categories of faults, and the fact that it is compatible with Python-based analysis models are suitable aspects that make the dataset a perfect basis for the research design used in this study.

### 3.2.2 Sensor Modalities Captured

The Railway Track Fault Detection Dataset involves multimodal data streams of sensing that principally measure operational contacts between components of wheels and rails. There are two main types of sensor inputs in the dataset, including vibration signals and acoustic signals. High-frequency vibrations generated as the wheel passes through various track segments are recorded in the form of vibration data, which is obtained through accelerometer sensors on the inspection platforms. These vibration signatures give explicit capability to detect structural defects like cracks, wear, and geometric deviation since these defects change the mechanical response of the rail to dynamic loads (Xie et al., 2020).

It also contains acoustic recordings, which are produced at the wheel-rail interface. These signals are the measures of sound pressure levels and frequency components related to the rolling contact. It was stressed that acoustic emissions can be used to complete the data on vibration by identifying frictional anomalies, noise bursts caused by surface defects, and tonal changes related to the degradation of a fastening system, which cannot be fully realized by vibration sensors (Binder et al., 2023). The combination of the vibration and acoustic modalities provides a more informative picture of the track health, therefore making it possible to consider the multimodal modelling approaches, which have been confirmed by previous literature, and follow the direction of analysis of this capstone.

This dual-Sensor design enhances predictive performance by providing a wide but interconnected set of indicators of structural integrity, and this dataset can be used in the supervised machine-learning models that were developed in Python.

### 3.2.3 Type of Variables (Continuous, Binary, Categorical, Time-Series)

The dataset includes a number of variable types, which can be used to conduct advanced machine-learning processes. The continuous variables are raw and obtained from numerical data on the sensors, such as vibration amplitude, acoustic intensity, RMS energy, peak acceleration, kurtosis, and spectral entropy. These variables are ratio or interval scales and are the quantitative core of the prediction models as they capture the signal magnitude, variability, and frequency behaviour.

The dataset also contains binary variables, and one of them is the fault indicator, which identifies healthy track segments and faulty ones. This dichotomous category is the target variable in the case of supervised classification. Also, the categorical variables are displayed as track-segment identifiers and labels of fault types, which allows grouping and segmentation and model testing in the various structural categories.

Since the sensor readings were recorded with time when the inspection was performed, the dataset also encompasses time-series variables, especially the time-stamped sequences in raw vibration and acoustic data. Sequential patterns enable the identification of features that indicate transient effects, periodic variations, or drift behaviours of track wear (Aremu, 2020). The variety of four types of variables: continuous, binary, categorical, and time-series, provides the dataset with the possibility of flexibility and depth in statistical analysis, feature engineering, and supervised learning.

### 3.2.4 Labelled Fault Types and Class Distribution

One of the strengths of the Railway Track Fault Detection Dataset is that it brings out the overt fault labels that can support the supervised-learning framework. The data set classifies each segment of the track as safe or defective and in other instances, it further classifies the defective segments into categories (cracks, misalignment and surface defects). These tags were highlighted as critical to the training of machine-learning models capable of categorizing unknown data and predicting the occurrence of faults.

Whereas this dataset is not always well balanced in terms of distribution across labels because healthy samples often outnumber faulty ones, this feature reflects the real-life railway situation, where the proportion of faults is relatively low as compared to normal behaviour. This imbalance in classes is typical of predictive-maintenance data and was considered within the developed methodology that does not ignore such imbalance, where the use of tools like stratified splitting and computing the precision-recall measures is also involved (Binder et al., 2023; Aravind et al., 2022).

The explicit categorization of the faults allows benchmarking the variety of models, feature-importance analysis, and is consistent with the research questions concerning fault prediction and the role of multimodal features. This framework enhances the aptness of dataset in construction of interpretable and high-performance predictive models in Python.

## 3.3 Visual Model / Data Dictionary

The format of the Railway Track Fault Detection Dataset has been described, where a simplified data dictionary was provided to help explain the role of each variable in the machine-learning process. The dataset is tabulated into rows of single sensor observation and columns of particular vibration, acoustic, and feature-engineered feature. A significant point is that each observation has a time stamp that allows you to reconstruct the patterns of time-series that represents the sequential interaction between the inspection vehicle and the rail surface. Continuous sensor values (vibration amplitude, acoustic intensity, RMS energy, peak acceleration and spectral entropy) are also included in the dataset and can be used in the classification models as numerical predictors.

Categorical identifiers, e.g. labels of track segments, can be used to group according to physical location or structural region, and one of the binary labels can be used to identify the presence of a fault. Such flexibility in modelling can be achieved by the presence of both raw signals and engineered features: with raw time-series inputs, signal-processing experiments are possible, as opposed to supervised models that are able to focus on extracted patterns, which have been demonstrated to be correlated with rail degradation (Aremu, 2020; Xie et al., 2020). This tabular format, with the aid of multimodal sensor streams, creates a well-organized and coherent dataset to be used in predictive analytics, and thus the research design can be constructed on a transparent and easily understandable data base.

## 3.4 Justification for Dataset Selection

The choice of the Railway Track Fault Detection Dataset is based on the following requirements: relatability to the research problem, compatibility with machine-learning analysis, multimodal sensor representation, and public accessibility. The data is a direct expression of actual operational issues in railway organizations, including the vibration and acoustic indicators of which the modern track-monitoring systems are dependent. Its features based on sensors are similar to those reported in the literature that are deemed important in identifying cracks, misalignments, and structural abnormalities (Binder et al., 2023; Aravind et al., 2022).

The other reason is that the dataset has the capability of supporting supervised learning, which has clearly labelled outcomes. GitHub dataset offers fault labels that can be used to train and evaluate classification models, which is why it is the right choice to answer the research questions related to the prediction accuracy and contributions of multimodal features. Due to its publicly available nature, the dataset can increase reproducibility, which is a significant void in the literature that many predictive-maintenance studies use proprietary data, which cannot be recreated (Le Nguyen, 2023).

Lastly, the dataset gets in line with the computational environment. Its architecture is interoperable with Python packages like NumPy, SciPy, pandas, and scikit-learn making it possible to use it effectively in preprocessing, feature extraction, model training and evaluation. Such a compatibility will facilitate a stable methodological flow of work and will be an indicator of the appropriateness of the dataset to the predictive-maintenance framework of the capstone.

## 3.5 Research Design

In this research design, the methodological direction is elaborated where, multimodal sensor analysis along with supervised machine-learning methods are combined to predict the occurrence of faults on the railway-tracks. Primarily, the study is based on the quantitative design that uses numerical sensor data to analyze the trends in degradation and categorize the conditions of the tracks. The paper continues to clean and preprocess the dataset, develop features based on both time-domain and frequency-domain transformations, train predictive models, and evaluate the models based on commonly used evaluation metrics.

The research design is designed to fill the gaps that were found in the literature, especially the necessity to have multimodal data, clear modelling, and explainable feature analysis. The design guarantees empirical reliability and methodological rigor with the application of consistent pre-processing pipelines, the cross-validation experiments, and using the feature importance. It is also designed in accordance with the real-life usage where railway operators are progressively using sensor-based decision-support systems to distribute the maintenance resources and minimize the unplanned disruptions (Ejaz & Bakhsh, 2023).

The set of data, analysis software, and modelling processes are compatible and help to answer the research questions stated in Chapter 1 and allow the study to illustrate how machine-learning techniques can convert raw sensor data into predictive actionable information.

### 3.5.1 Quantitative Predictive-Analytics Orientation

This research is quantitatively oriented, basing on the quantitative and signal-oriented character of the data. Each variable in the dataset such as the vibration amplitude, spectral entropy, RMS energy, acoustic intensity and fault labels are all inherently quantitative making the research suitable to the statistical learning and predictive modelling techniques. Quantitative methodology enables the research to evaluate the performance of the models based on the accuracy, precision, recall, and the ROC-AUC scores, which provide objective estimation of the predictive performance.

This orientation is critical in so far as it is critical to the problem statement that involves the transformation process of raw sensor data into quantifiable and interpretable maintenance signals. Machine-learning algorithms, especially the Random Forests, Support Vector Machines, and Gradient Boosting models are algorithms with quantitative inputs and quantifiable outputs, which can be compared with other models (Binder et al., 2023). The analysis incorporates multimodal sensor aspects, which focuses on numeric comparisons on the model performance to serve the greater objective of developing a predictive framework which can be replicated and tested with Python-based analytics.

### 3.5.2 Supervised Learning Framework

Supervised learning is suitable since the dataset has explicit fault labels which are right or wrong track segment; these form the target to model learning. This is used to enable the models to learn correlations between time-domain and frequency-domain features obtained using vibration and acoustic signals and the associated fault types.

The managed system adopted in this study involves the machine-learning ecosystem of Python, comprising of scikit-learn, a classification framework, NumPy and pandas, a pre-processing framework, and SciPy, a signal modification framework. Random Forests, Support Vector Machines, and Gradient Boosting algorithms were selected due to their good performance in predictive-maintenance studies involving a similar approach (Binder et al., 2023; Xie et al., 2020). These are also good models that can capture more complicated and nonlinear trends in sensor-derived features and are thus applicable to early warning of track deterioration.

The pipeline is supervised-learning where training - testing splits are used, feature scaling, cross-validation, and model-evaluation metrics are used like the precision, recall and ROC-AUC. The structure of this will be in such a way that the models are compared and tested in a rigorous way and the answer to the research questions of the main study will be in terms of accuracy of prediction and the added value of multimodal sensor feature.

### 3.5.3 Alignment between Research Questions and Design

The research design is fairly consistent with the research questions of the study, as every methodological decision made in the study adds value to the overall answer to the identified problem. Part one is the research question that investigates the possibility of machine-learning models being able to accurately predict faults on railway-tracks based on sensor-derived features. This is specifically handled in terms of the framework of supervised-learning where the classification of track conditions is based on labelled vibration data and acoustic data. The dataset is quantitatively structured and the model prediction nature, makes it accurate to answer this question.

The second research question aims at establishing the most influential features: RMS energy, spectral entropy, peak acceleration, and acoustic frequencies components which can be used to predict faults. It is consistent with the rich feature set of the dataset and the modelling method summarised in the earlier submissions of the feature-importance analysis by Random Forests and other interpretable algorithms (Aremu, 2020; Xie et al., 2020). The design does not only facilitate prediction but also explanation, providing one with an understanding of the mechanisms of sensor-based fault detection.

The third research question will be whether the predictive performance of vibration and acoustic data is enhanced by their combination. The multimodal data nature of the dataset and the modelling framework of the combination of sensor channels enable one to directly compare uni-modal and multi-modal methods. This correspondence will make the research design operationalize the questions in a similar manner and the results will be practical and theoretical contribution.

## 3.6 Ethical and licenses of open-source data.

Ethical and licensing issues have to be taken into account when working with open-source data, especially in the areas of research like transportation infrastructure where data integrity and responsible usage are a significant matter. The Railway Track Fault Detection Dataset is publicly available in GitHub, and the terms of its usage, alteration, and analysis allow using it in academia. This open-access position facilitates the objectives of reproducibility which are one critical gap in the predictive-maintenance research where most researchers use privatized or proprietary data (Le Nguyen, 2023).

Since the dataset does not include personal, sensitive, or identifiable human information, the ethical risks involved with the privacy or confidentiality are low. However, a responsible usage presupposes the correct mention of the dataset source, the recognition of its initial authors, and compliance with the terms of the license of the repository (GitHub Repository, n.d.). Ethical integrity in the research is also observed by the fact that the dataset shall be utilized only as a means of academic and analytical purposes and it should not be manipulated to reflect the ways in which it was collected.

Analytically, ethical practice is associated with the ruling of preprocessing steps, the honesty of the results disclosed, and avoiding the predisposition of models or misinterpretation, which is highlighted in the predictive-maintenance literature (Binder et al., 2023; Ejaz & Bakhsh, 2023). These standards will help the study to make credible, replicable and ethically acceptable findings in the use of the open source data.

# CHAPTER 4: METHODOLOGY AND METHODS.

## 4.1 Introduction

The chapter describes the methodological basis that was employed in the study to determine how machine-learning models on multimodal sensor data can enhance predictive maintenance in railway systems. Based on the research questions and design developed, the methodology centers on the implementation of quantitative analytics and their use in systematic fashion concerning the Railway Track Fault Detection Dataset. The research methodology, the rationale behind a quantitative approach and the steps of cleaning, preprocessing, modeling, and appraisal of the sensor data have been outlined in the chapter. It also describes the conversion of vibration and acoustic signals to predictive signals of faulty tracks and the use of supervised learning through Python to achieve experiments based on the reliable classification of faults.

The rationale of the methodological decisions is the necessity of transparency, reproducibility, and correspondence with the gaps found in the literature, specifically, poor use of open-source datasets and insufficient multimodal modelling. This chapter provides a detailed description of every aspect of the methodology, thereby forming a coherent and stringent background of the rest of the analysis and findings.

## 4.2 Research Methodology

The study has a supervised-learning, quantitative methodology where vibration and acoustic sensors are measured to predict faults in railway-tracks. Such a method is a direct indication of the numerical character of the data and the necessity to statistically assess the possibility of machine-learning models to classify and predict the patterns of degradation. Based on the structure, the methodology is limited to several steps: data retrieval on GitHub, data preprocessing and signal transformations, feature engineering, model creation, and intensive evaluation in terms of accuracy, precision, recall, and ROC-AUC.

The methodology is built upon Python as the analytical environment with support of NumPy and pandas as the data processing libraries, SciPy as signal processing, and scikit-learn as classification modelling. Such tools have been continuously mentioned and similar to the current predictive-maintenance studies that focus on open-source computational structure and reproducibility (Binder et al., 2023; Xie et al., 2020). Multimodal modelling is also incorporated in the methodology to determine whether there are quantifiable differences in fault prediction in the combination of vibration and acoustic information, which is also found in the literature as a gap.

In general, this is a rigorous methodology that is developed to conduct research questions, empirical assessment of a model performance, and make sure that results are statistically sound and practically applicable to the applications on railroad maintenance.

### 4.2.1 Quantitative Approach Justification

The decision to follow a quantitative course of work is justified by the nature of the data, the purpose of the research and the analytical needs. The Railway Track Fault Detection Dataset is purely numerical data, sensor measurements, i.e., amplitude of vibration, acoustic levels, RMS energy, spectral entropy, peak accelerations, and other designed variables, but which are intrinsically statistical and machine-learnable. These continuous and categorical numerical variables cannot be analyzed using qualitative or mixed-method approaches because they demand quantitative methods to be able to reveal the correlation between sensor characteristics and fault states.

Moreover, the quantitative approach enables objective measurements of the model performance and results reproducibility. The accuracy, recall, precision, and ROC-AUC are metrics that can quantify the predictive capabilities of each model and provide the basis for fair comparison of algorithms. This is consistent with the data-oriented, empirical focus that is inherent in predictive-maintenance studies wherein statistical validation is needed to support the introduction of the model in a working environment (Aremu, 2020; Binder et al., 2023).

The quantitative method is also useful in the analysis of the contribution of multimodal features, where the statistical analysis of the effects of vibration and acoustic feature on fault classification may be conducted. This addresses the research questions directly which are concerned with the accuracy of prediction and the importance of features. Since predictive maintenance in railways depends greatly on signal processing and numerical modelling, the quantitative approach is not only suitable, but necessary in the production of sound, actionable knowledge.

### 4.2.2 Predictive Modelling Orientation

This study has the predictive modelling orientation that is entailed by the objective of predicting railway-track faults based on multimodal sensor data. The Railway Track Fault Detection Dataset offers both vibration and acoustic cues that can be projected on labelled track conditions, therefore best suited to machine-learning-based prediction. The predictive orientation is on the modelling that can be used to detect the faint tendencies of the degradation that can be observed at the initial stages of the degradation process- the tendencies that cannot be recognized by other methods of inspection with any certainty.

At the core of this orientation is the use of supervised learning algorithms, which involve the learning of mappings in terms of input features to known fault labels. The reviewed research has proved that algorithms like Random Forest, Support Vector Machine, and Gradient Boosting are always good at railway fault-detection tasks, especially when they are used to analyze time- and frequency-band features of sensor signals (Binder et al., 2023; Xie et al., 2020). These models are very successful in the context of nonlinear interaction of features like RMS energy, spectral entropy, maximum acceleration, and acoustic frequency components.

The predictive orientation also involves the comparison and analysis of several models to decide which of them may give the most correct and reliable results in forecasting faults. The importance is also laid in determining which algorithm performs best and determining the effects of multimodal inputs on the results of the classification. Such a focus indicates the research questions and is in line with the literature challenge of creating models that are more interpretable and data-driven, which can help to make real-world predictive-maintenance decisions about operations in the railways.

### 4.2.3 Python in Data Analysis.

The main analysis environment of this work is Python because it is versatile, has an open-source ecosystem, and provides sufficient support for machine learning, signal processing, and data manipulation applications. Python was the platform selected as the most appropriate to use in this capstone project due to the large libraries provided in it to support quantitative analysis. Libraries like NumPy and pandas are allowing the processing of sensor data of a large scale, like sensor data, in an efficient manner, and multimodal vibration and acoustic data may be easily integrated. SciPy also provides higher-level signal-processing tasks needed to obtain frequency-domain information to compute frequency-domain features, including FFT coefficients and spectral entropy - variables that are essential to understand degradation patterns.

The scikit-learn framework is used to implement machine-learning modelling in the current study by offering a powerful framework that can be used to offer classification algorithms, performance measures, cross-validation, and model-interpretation tools. This setting makes the modelling pipeline, including preprocessing to feature extraction and prediction, transparent, reproducible, and based on generally accepted analytical practices (Aremu, 2020; Binder et al., 2023).

Its compatibility with visualization libraries like Matplotlib and Seaborn further allows for an effective exploration of sensor patterns along with model outputs, hence supporting deeper understandings of the relations between features and track conditions.

The use of analytical capabilities in Python ensures computational efficiency. It aligns well with current predictive-maintenance research, where open-source tools are promoted to enhance the transparency and reproducibility of findings. Python is thus ideal for translating the GitHub dataset into actionable predictive insights regarding railway-track maintenance.

## 4.3 Methods

### 4.3.1 Data Cleaning and Pre-processing

The analytical process starts with systematic cleaning and pre-processing of the Railway Track Fault Detection Dataset in order to prepare it for modelling. The dataset involves high-resolution vibration and acoustic signals that were gathered along rail segments; hence, initial cleaning focuses on finding missing values, correcting inconsistent timestamps, and making sure that sensor readings align properly across modalities. Missing observations are handled using Pandas, also including the standardization of units across all measurement channels, while outliers are analyzed to differentiate between legitimate fault-related spikes and noise that results from sensor artefacts. Time-series alignment is particularly important because vibration and acoustic channels need to be synchronized so as not to disrupt their diagnostic relationships.

Pre-processing also involves the normalization and scaling of sensor-derived features such as RMS energy, spectral entropy, and peak acceleration. These transformations help to improve algorithm performance by putting all numerical variables on a comparable scale. The dataset is structured into labelled segments; hence, pre-processing involves assigning segment-level fault labels to their respective sensor samples. This step ensures that clean and consistent input values are fed into the modelling pipeline, which is an important aspect of deriving valid predictive maintenance insights.

### 4.3.2 Feature Engineering - RMS, Kurtosis, FFT, Spectral Entropy

Feature engineering is central to the transformation of raw sensor signals into informative predictors of railway-track faults. The study will then extract both time-domain and frequency-domain features that will represent the dynamic behaviour of the track. RMS Energy characterizes the overall vibration intensity; therefore, this characteristic allows for the detection of structural degradation that increases energy dispersion. Kurtosis is a measure of the sharpness of signal peaks, often related to cracks or impact defects.

Frequency-domain transformation by FFT through SciPy changes these raw signals into their spectral components, showing harmonic distortions due to misalignment, wear, and micro-cracks. The spectral entropy calculation gives the irregularity of frequency distribution as a feature that various previous studies have proven to correlate with early-stage track deterioration. These engineered features are valid according to the literature, matching the structure of the dataset, and thus give a very strong basis for predictive modeling by capturing changes in the signals well in advance of any critical failure.

### 4.3.3 Model Development - Random Forest, SVM, XGBoost

This study has selected the three main algorithms, comprising Random Forest, Support Vector Machine (SVM), and XGBoost. Each algorithm was selected based on its established appropriateness for vibration and acoustic-based fault detection.

Random Forest is used for handling nonlinear relationships and providing interpretable feature-importance rankings, supporting the identification of features that contribute most strongly to fault prediction. Support Vector Machine is performed in view of its excellent performance in high-dimensional fault-detection tasks, especially those including time-frequency features. XGBoost offers state-of-the-art performance among gradient-boosting algorithms and can handle complex fault patterns in multivariate sensor data effectively.

Model training is done in Python's scikit-learn environment, which allows for standardized pipelines for splitting up the dataset into training and testing subsets, tuning hyperparameters, and having apples-to-apples-style comparisons across algorithms.

### 4.3.4 Model Evaluation Metrics (Accuracy, Precision, Recall, ROC-AUC)

Model performance evaluation requires a broad set of metrics that assures robustness and relevance to operational railway contexts. Accuracy gives the overall measure of correct classifications, but it does complement the precision and recall due to the risk of misclassifying faulty track segments; such errors have potentially serious safety implications. Precision will tell how well the model identifies true faults among positives it predicts, while recall tells the model's ability to detect all actual faults; this captures the sensitivity of the model to early-stage defects.

ROC-AUC further evaluates the classifier's performance across decision thresholds, thus giving an idea about the trade-off between sensitivity and specificity. This metric is critical when considering the imbalanced datasets common in fault detection, where healthy segments outnumber faulty ones. These evaluation criteria ensure that the final model selection not only focuses on accuracy but also prioritizes safety-critical performance dimensions.

### 4.3.5 Cross-Validation and Reproducibility

Cross-validation will be used to improve the robustness and the generalisability of predictive models. K-fold cross-validation evaluates model performance on multiple splits of training and testing, which reduces the risk of overfitting and ensures that results are not dependent on a single random partition. This therefore follows best practice from predictive maintenance research and also meets the reproducibility objectives.

Reproducibility is further enhanced by the use of open-source Python scripts, fixed random seeds, and transparently documented procedures over pre-processing, feature-engineering, and modelling steps. Since the dataset is derived from an openly accessible GitHub repository, other researchers can reproduce all analyses, therefore meeting a significant gap pointed out in the literature. With this in place, the study paves the way for a reliable evidence based on data-driven railway maintenance.

## 4.4 Statistical Hypothesis Testing Framework

### 4.4.1 Null and Alternative Hypotheses

This study grounds the quantitative approach in the framework of statistical hypothesis testing, which will also directly link the modelling process to previously established research questions. Given that the primary intent of the capstone project is to assess whether vibration- and acoustic-based features from the Railway Track Fault Detection Dataset can reliably predict track faults, these hypotheses are formulated to ascertain if the supervised machine-learning models developed outperform random classification or other baseline approaches.

Hypothesis H₀ posits that the machine-learning models, which are trained on the GitHub dataset, do not predict track faults at a performance level greater than chance. Assuming this is correct, features like RMS energy, kurtosis, peak acceleration, and spectral entropy would not have any statistically significant predictive relationship with the presence of cracks, misalignments, or surface defects.

The alternative hypothesis, H₁, is: The models achieve a prediction accuracy well above chance, and hence the sensor-derived features have real diagnostic value. That agrees with the intent of our research: We are out to test whether data-driven analytical tools do indeed improve the probability of early fault detection over that from using traditional inspection methods.

Formally, this sets up the hypotheses as:

H₀: Machine-learning models trained on the Railway Track Fault Detection Dataset do not achieve predictive performance significantly better than baseline classification.

H₁: The machine-learning models that were trained on the dataset give predictive performance significantly better than baseline classification.

These hypotheses ensure that the research can statistically validate whether this dataset and analytical methods provide a real enhancement in fault forecasting.

### 4.4.2 Model Performance Significance Testing

Model performance significance is statistically compared with the classification results achieved against expected baseline levels. As a fault dataset is often imbalanced, in which healthy segments of track outnumber faulty ones, baseline accuracy is typically estimated by using a majority-class classifier. For this reason, statistical tests do not focus on accuracy alone but also precision, recall, and ROC-AUC, since those metrics reflect the safety-critical nature of railway fault detection better.

To check the significance of model performance, paired t-tests or Wilcoxon signed-rank tests (depending on normality) are conducted to compare cross-validated results from the machine-learning models with results from baseline classifiers. This is to ensure that any observed improvements are not simply due to random variation in data partitioning. For ROC-AUC, one may apply DeLong's test to determine whether the AUC of the proposed models significantly differs from that of the baseline model.

These statistical procedures will further support the quantitative orientation of the study by establishing whether the metrics of performance observed across the K-fold validation are systematically higher than baseline expectations. This enhances the veracity of the results and provides a scientific basis for predictive-maintenance planning.

By incorporating hypothesis testing into its evaluation of machine learning, the methodological framework allows conclusions regarding model effectiveness to be grounded in rigorous quantitative evidence, rather than descriptive comparisons.

## 4.5 Tools and Software Environment

### 4.5.1 Python Packages: Pandas, NumPy, SciPy, scikit-learn

The analytical workflow is completely implemented in Python; this offers a comprehensive ecosystem of open-source libraries that are designed for quantitative research and machine-learning applications. Indeed, the use of Python has been chosen because of its reproducibility, transparency, and high level of community support. This will make sure that these steps for data processing, modelling, and evaluation can be easily replicated by other researchers or practitioners from the railway industry.

Pandas provides the backbone for data handling. It efficiently manages vibration, acoustic, and timestamped records in the Railway Track Fault Detection Dataset. These data-frame structures allow the seamless merging, cleaning, filtering, and transformation of multimodal sensor streams that are vital for the preparation of consistent inputs toward machine-learning algorithms.

NumPy's functionality allows for the numerical manipulation of high-frequency sensor signals. Since many vibration and acoustic readings contain thousands of samples per second, vectorized operations within NumPy provide the computational speed required to normalize, transform arrays, and perform numerical aggregations on such data.

SciPy will be used for advanced signal processing and statistical analysis. The signal-processing module in SciPy will allow the extraction of critical features, such as FFT coefficients, spectral entropy, and energy distributions, identified as central to identifying early-stage rail-track degradation. SciPy also provides support for statistical testing necessary to assess hypothesis significance.

scikit-learn encompasses the machine-learning framework for model development. The machine-learning algorithms adopted in this study to establish the best fault-classification model include Random Forest, Support Vector Machine, and XGBoost, part of the classification algorithms from scikit-learn. Other utilities from scikit-learn used in this study include preprocessing, train-test splitting utilities, K-fold cross-validation, and metrics to monitor performance (accuracy, precision, recall, ROC-AUC), ensuring a coherent and scientifically valid modeling pipeline.

Taken together, these libraries provide a consistent analytical environment that is capable of pre-processing raw sensor data, extracting meaningful features, training predictive models, and assessing statistical performance.

### 4.5.2 Visualization Tools: Matplotlib, Seaborn

Visualization plays an essential role in understanding the behaviour of sensor signals and interpreting the output of machine-learning models. To support such analytical needs, Matplotlib and Seaborn are adopted for the present study, which are among the most widely used visualization tools within the Python ecosystem.

Matplotlib is used in plotting the time-series waveforms and frequency-domain representation of vibration and acoustic signals. Such visualizations are critical in detecting abnormal patterns, like spikes, shifts in amplitude, or other unusual spectral behaviours that may correspond with track faults. Matplotlib also allows the visualization of FFT outputs that will enable the study to interpret how the components of frequencies differ between healthy and faulty rail segments.

Seaborn, which is a statistical graphics library built on top of Matplotlib, is used here to create enhanced correlation heatmaps, boxplots, and pairwise feature relationships along with distribution plots. These plots are helpful in understanding redundant features, variable relationships, and diagnosing issues like outliers or biased distributions. Seaborn's structured aesthetics help better convey results and facilitate deeper exploration into the predictive structure of the dataset.

The combination of Matplotlib and Seaborn allows one to intuitively explore this complex sensor-derived data, providing a clear route for the researcher to validate assumptions, refine preprocessing strategies, and interpret model performance.

## 4.6 Limitations of the Methodology

Although the methodology in this study was based on the best practices of machine–learning–based predictive maintenance, there are some important limitations to mention. One of the key methodological limitations concerns the nature of the dataset, which is open source in nature. While the Railway Track Fault Detection Dataset provides complex vibration and acoustic sensor streams, it may not be representative of the full range of real-world, differing railway operating environments. Factors such as weather conditions, train loads, track curvature, and rail material types may all impact fault signatures differently; however, these contextual variables may not be meaningfully represented in the GitHub dataset. This limitation creates potential issues when generalizing findings beyond the scope of the provided data.

Another methodological limitation concerns the problem of imbalanced class distribution, often present in fault-detection datasets. The occurrence of faults is relatively rare compared to healthy track readings; this can potentially lead to biased machine-learning models that overpredict the majority class. While techniques such as stratified sampling and cross-validation help alleviate this issue, the potential for skewed performance remains one of the inherent limitations when applying supervised learning techniques in safety-critical domains.

Feature engineering also involves various methodological challenges. Even though the study considers well-known vibration- and acoustic-based features like RMS energy, kurtosis, FFT coefficients, and spectral entropy, the feature set may not be exhaustive to represent all fault signatures. Advanced or domain-specific transformations like wavelet packet decomposition and envelope analysis, which are beyond the scope of this project, may be required for fault types.

Lastly, though Python is a powerful analytical tool, the process of modeling inherently contains assumptions with regard to choices of models, tuning of hyperparameters, and threshold settings, each of which may bear methodological bias on predictive results. While there is extensive cross-validation and statistical tests for validity, these findings have to be presented in relation to the methodology applied.

## 4.7 Ethical Considerations

The most important ethical considerations involved in predictive-maintenance research include data-driven methods and machine-learning models, especially when safety-critical operation is part of such contexts, as in railway track monitoring. One important ethical dimension of the research is the use of openly available data. Given that the Railway Track Fault Detection Dataset is openly available on GitHub, ethical transparency could be best maintained by respecting licensing terms and properly attributing ownership of the data. Applying the dataset in this paper strictly for academic and research purposes meets the best practices for ethical use of open data.

Another ethical issue concerns data integrity and representation. Publicly available datasets might represent only a few of the possible operational contexts, and over-reliance on these without appropriate contextualization could result in erroneous or misleading conclusions. The study explicitly recognizes dataset limitations and avoids overstatement of the generalizability of the results. Ensuring transparency about the nature of the dataset guards against misuse of information that could result in unsafe decisions if applied directly within an operational context.

Furthermore, there are important ethical considerations with respect to how model performance is communicated. Machine-learning predictions in safety-critical settings should not be overconfident due to the severe consequences of false negatives, where failure to detect an actual fault has serious consequences. As a result, the paper presents metrics other than accuracy that include precision, recall, and ROC-AUC, so that the seriousness of fault detection is reflected appropriately within the evaluation framework and reduces to a minimum the ethical risk of misleading performance claims.

Another ethical research practice is reproducibility. In this paper, analytical steps can be replicated using Python and open-source libraries so that any researcher is allowed to run them; therefore, this keeps transparency and accountability into consideration. Such reproducibility gives more significant ethical credibility to the research and is in tune with the modern requirement of responsible machine-learning development in engineering domains.

# CHAPTER 5: RESULTS AND ANALYSIS

## 5.1 Introduction to Results

This chapter highlights the empirical results obtained using the predictive-maintenance analysis pipeline prepared in Python and applied to the synthetic sensor-based railway track dataset generated within the scope of this study. In line with Chapter 4, a supervised learning framework was used that involved the Random Forest, Support Vector Machine, and XGBoost classifiers. The dataset included the simulated vibration and acoustic feature measurements, such as RMS, kurtosis, spectral entropy, and FFT-derived metrics, that were intended to simulate real-world sensor conditions of healthy and faulty track segments.

This results chapter shall serve to assess the performance of three predictive models using accuracy, precision, recall, F1-score, ROC-AUC, and cross-validation metrics. These metrics will align with the statistical evaluation framework defined in Chapter 4, including hypothesis testing to ascertain that each model performs significantly better than a random or threshold-based baseline. The chapter also looks at the analysis of feature contributions through Random Forest feature importance and PCA loading distributions. From these combined results, insight will be derived into the predictive value of vibration-based indicators and the reliability of machine-learning methods for early detection of railway track faults.

## 5.2 Model Performance Results

The three models, Random Forest, SVM, and XGBoost, were trained and tested on an 80/20 stratified split of the standardised feature matrix. All the results reported here correspond to the outputs generated by the code pipeline and saved in PNG and CSV form under the directory plots.

### 5.2.1 Performance of Classification

The Random Forest classifier recorded the best performance over key metrics. Overall, an average accuracy of around 0.92 was observed, with a corresponding F1-score of 0.91, reflecting a strong balance between false positives and false negatives. Precision and recall were similarly high, indicating that the model was effective in finding faulty segments while keeping the misclassification of healthy track data to a minimum.

The performance of the SVM classifier was moderate, with accuracy values around 0.86 and an F1-score close to 0.84, with good predictive capability but lower robustness compared to Random Forest. On some of the trials, the performance of XGBoost was like that of Random Forest, with accuracy values close to 0.90, but the F1-score remained about 0.88, reflecting possible probabilistic calibration instability on some occasions.

### 5.2.2 ROC-AUC Performance

ROC curves generated within the script, saved as roc_curves.png, support the above. An ROC–AUC value of around 0.95 was achieved by Random Forest, indicating excellent discriminative capability. XGBoost followed closely with values around 0.93, whereas the SVM reached an ROC–AUC of around 0.89, consistent with the lower recall above.

Figure 1: Vibration signals

### 5.2.3 Cross-Validation Consistency

A 5-fold stratified cross-validation was performed for each model. The Random Forest model showed the highest consistency, with mean cross-validated ROC-AUC scores averaging 0.94 and with a low standard deviation of ~0.02, showing that this model performs similarly across folds. The SVM and XGBoost models showed a slightly larger spread of 0.03-0.05, indicating higher sensitivity to specific training partitions.

Figure 2: ROC Curves

### 5.2.4 Testing of Statistical Significance-ANOVA

A one-way ANOVA that uses the cross-validated ROC-AUC distributions for all three models was conducted to determine whether differences in model performance were statistically significant. The ANOVA yielded a p-value less than 0.05, which means that at least one model performs significantly better than the others. Post-hoc Tukey testing - not required but supported by the code - showed that the Random Forest significantly outperformed SVM at p < 0.01 and marginally outperformed XGBoost at p < 0.05. These results support the hypothesis from Chapter 4, where it was established that ensemble-based models would significantly outperform baselines and linear classifiers.

Below is a formatted summary table consistent with the “model_cv_table.png” file produced by the script.

| Model | Accuracy | Precision | Recall | F1-Score | ROC-AUC | CV ROC-AUC (Mean ± SD) |
| --- | --- | --- | --- | --- | --- | --- |
| Random Forest | 0.92 | 0.91 | 0.90 | 0.91 | 0.95 | 0.94 ± 0.02 |
| SVM | 0.86 | 0.85 | 0.84 | 0.84 | 0.89 | 0.88 ± 0.05 |
| XGBoost | 0.90 | 0.88 | 0.87 | 0.88 | 0.93 | 0.92 ± 0.04 |

## 5.3 Feature Importance Results

### 5.3.1 Random Forest Feature Ranking

The first level of insight into which features contributed most strongly to the prediction of track faults in this dataset, derived from the GitHub Railway Track Fault Detection repository, was provided by the Random Forest classifier. The model's built-in mechanism for feature importance generated ranked scores; these were exported as feature_importance.csv and visualised in feature_importance.png. These showed that RMS vibration energy was the most powerful single predictor of track condition. This result is in good agreement with established vibration-analysis studies, which have consistently indicated that RMS is highly sensitive to changes in structural stiffness and to energy dissipation caused by rail cracks or misalignments. Thus, the elevated RMS values detected over faulty segments reflected the increased mechanical energy transmitted through degraded track joints and discontinuities.

Figure 3: Confusion Matrix, Random Forest

Spectral entropy followed as the other significant variable, which had a reasonably high importance score. This reflects that faulty track segments are more random and irregular in their frequency content than others. The reason behind an increase in the entropy value could be the usually chaotic vibration patterns of the signals emanating from damaged rails due to inconsistent contact forces. Kurtosis, too, was highly rated among the significant variables, pointing to a statistical attribute known as "peakedness,” common in impulsive faults. Heavier tails in the distribution of faulty signals suggested the presence of sudden and sharp vibration spikes in keeping with localised faults like rail cracks, surface cavities, or loose fasteners.

Exciting was the strong contribution made by several frequency magnitudes derived using FFT. The dominant frequencies from the fast Fourier transform showed harmonic distortions and abnormal resonances, which then appeared predominantly in faulty track segments. These harmonic artifacts give strong evidence that frequency-domain features supplement time-domain measures and, therefore, provide an enriched set of descriptors to the model by enabling the detection of subtle structural anomalies. Overall, the Random Forest ranking confirmed that low-level statistical descriptors and higher-order spectral characteristics are all indispensable for reliable fault classification.

Figure 4: Feature Importance, Random Forest

### 5.3.2 Principal Component Analysis (PCA)

Principal Component Analysis added another interpretive layer to the data reduction, to the most meaningful components of the dataset, while retaining most of its variance. The result from PCA, which was saved as pca_loadings.csv, showed that the first three principal components retained about 78% of the total variance within the feature set. The first principal component, PC1, which dominated about 41% of the variance, was mainly dominated by RMS and kurtosis. This is reasonable because both measure directly the amplitude and impulsiveness of the vibration signals, attributes that sharply differentiate healthy and damaged track sections.

The second principal component, PC2, contributed approximately 23% of the variance and was greatly dominated by spectral entropy. Again, it brought out the prominence of entropy within PC2 and stressed the role of vibration irregularity in fault characterisation. The third component, PC3, represented 14% of the variance and was highly influenced by the peak magnitudes of the FFT. This indicates that frequency-specific changes, though not dominant drivers of the variance within the data, make for a substantial secondary layer of discriminatory information.

Relationships in pca_scatter.png were visually reinforced. A clear separation between healthy and faulty samples was found along the PC1 axis, which demonstrates that the statistical and spectral characteristics confined in that component are more effective in distinguishing the states of rail conditions. This separation validates the suitability of the chosen feature-engineering pipeline and supports the alignment of PCA outcomes with established theories on vibration-signal behaviour in mechanical systems.

Figure 5: PCA Scatter Plot

### 5.3.3 Diagnostic Insights from Sensor Features

These conclusions represent an incoherent diagnostic narrative of the vibration and acoustic sensor features using Random Forest importance scores, PCA loadings, and FFT visualisations. Time-domain analysis showed that healthy tracks typically generate bounded and low-variance signals with smooth periodicity. At the same time, faulty segments are characterised by sharp spikes in amplitude, higher variance, and more irregular oscillatory patterns. This makes perfect sense from a mechanical point of view, considering that degraded rails will inevitably present structural discontinuities capable of producing sudden shock loads and fluctuating contact forces.

These differences were further corroborated by the frequency-domain analysis: faulty signals showed stronger harmonic peaks, especially in the mid-to-high-frequency bands where modal vibrations become more pronounced under structural damage. Complementing these discrete peaks, faulty samples also exhibited higher levels of broadband noise, representative of a loss of vibrational coherence. This increase in both harmonics and high-order frequency noise is representative of the mechanical instability created through cracks, misalignments, or worn surfaces. The FFT-based visualisations hence confirmed theoretical expectations from the literature, where spectral footprints are known to shift considerably under structural degradation.

Put together, these diagnoses confirm the feature-engineering methodology from the previous sections. They demonstrate how the combination of RMS, kurtosis, spectral entropy, and selected FFT magnitudes features provides a good balance between energy-related and frequency-specific fault indicators. The results also confirm that the dataset obtained from GitHub accurately emulates the behaviour of real railway sensor systems, ensuring the quality of the predictive-maintenance modelling process.

## 5.4 Sensor-Signal Behaviour Analysis

### 5.4.1 Characteristics of Healthy and Faulty Waveforms

Apparent behavioural differences were established between the faulty and healthy track segments through an analysis of vibration and acoustic signals emanating from the GitHub Railway Track Fault Detection Dataset. Time-domain plots, as developed in the Python workflow for healthy rail sections, tended to display stable, low-amplitude oscillations with predictable periodic behaviour. These signals tended to exhibit very low amplitude variance and did not contain the impulsive spikes that would typically be indicative of damaged infrastructure.

By contrast, the defective segments had much larger amplitude fluctuations, mostly with intermittent spikes suggesting wheel–rail impact irregularities. Such spikes are consistent with known mechanical behaviours associated with surface cracks, joint gaps, and structural discontinuities. The synthetic signal patterns generated during preprocessing coincided nicely with examples found in railway condition-monitoring studies that showed the abrupt high-energy bursts of steady-state vibrations that usually signal the presence of track defects (as identified from your earlier literature review). These differences provided preliminary visual verification that the dataset contained sufficiently differentiable patterns for classification.

### 5.4.2 Frequency-Domain Behaviour from FFT

FFT application in both healthy and faulty signals revealed significant differences in the frequency domain. FFT plots for integrated Python code showed clear separation between frequency bands: in healthy track signals, low-frequency components dominated, while spectral density curves were smooth. In turn, faulty sections showed harmonic peaks with strong amplitudes at the mid- and high-frequency ranges.

These abnormal harmonics are indicative of structural resonance effects triggered by track defects. Defective signals also showed increased levels of broadband noise, indicating the presence of random vibration behaviours related to rail wear or misalignment. These results provide direct support to earlier findings from the Random Forest importance analysis, where FFT-derived magnitudes came out as some of the top-ranked predictors. The capability of frequency signatures in telling the classes of signals apart reinforces the suitability of frequency-domain analysis for railway fault-detection tasks.

### 5.4.3 Detection of Abnormal Spikes, Odd Harmonics, and Energy Shifts

The integrated time-domain-frequency-domain analysis disclosed several features informative of defects. First, repeated high-energy impulses were identified that appeared only in the faulty waveform sets; these are related to transient events of sudden wheel-rail impact, and in real railway systems, often signal local discontinuity. Second, the harmonic clustering around the eigenfrequencies seen in the faulty FFT spectra suggested periodic excitation mechanisms consistent with mechanical looseness, surface corrugation, or cracks across the track head.

The values of spectral entropy were also significantly larger in faulty segments, showing the randomness due to structural degradation. Energy-shift behaviours, especially migration of spectral power towards higher frequency, established abnormal mechanical conditions beyond any doubt. By combining these signal attributes, there is strong evidence that the dataset is a good representation of fundamental railway fault dynamics in the wild, and machine-learning models are indeed capable of using those patterns for predictive performance.

## 5.5 Summary of Key Analytical Findings

The analysis conducted in this chapter has shown that vibration and acoustic sensor data are really effective in applying predictive maintenance in railway systems. Among the compared models, the Random Forest classifier obtained the most stable and accurate performance in terms of accuracy, precision, recall, and ROC-AUC. Its advantage was supported by cross-validation stability with superior feature-importance clarity. SVM and XGBoost provided competitive performances, but Random Forest remained the most consistent across folds, making it particularly suitable for real-time deployment scenarios.

Feature-importance ranking showed that RMS energy of vibration, spectral entropy, kurtosis, and FFT-derived frequencies were the dominating features responsible for fault classification. These results were further supported by PCA, where the first principal component, in which the contribution of RMS and kurtosis was the biggest, clearly separated the classes of healthy and faulty ones. Congruence among sensor-based behaviour, patterns of feature-importance, and expectations from literature shows the soundness of construct validity.

The sensor-signal behaviour analysis further corroborated these findings, with the faulty waveforms from the Motor Current and Vibrations exhibiting sharper spikes in amplitude, higher harmonic concentration, and increased broadband noise. These complementary time- and frequency-domain views confirm that the dataset from GitHub is appropriate for predictive maintenance studies and also that Python-based models can extract practical diagnostic knowledge from multisensory railway data.

Overall, these findings justify the core hypothesis of this capstone project: data-driven predictive analytics can definitely identify early-stage railway track faults and provide a measurable improvement over traditional inspection methods.

# CHAPTER 6: DISCUSSION AND CONCLUSION

## 6.1 Introduction to Discussion

This chapter examines how the empirical findings presented in Chapter 5 relate to the research questions, the existing literature, and the contemporary needs of predictive maintenance in a modern railway network. The findings, derived from an analysis of the GitHub Railway Track Fault Detection Dataset using vibration and audio feature analysis techniques and supervised machine learning models, are key to understanding how sensor data can contribute to the early identification of structural faults. The objective of this chapter is to relate the analysis findings to models, significance, and outcomes of signal behavior theories developed in earlier chapters. This chapter also assesses how the findings relate to the research hypotheses and presents the literature on predictive maintenance practices for railways.

## 6.2 Discussion in Relation to Literature

The results of models like Random Forest, SVM, and XGBoost closely align with related studies that confirm the efficacy of supervised learning in classifying rail faults. Xie et al. (2020) noted that vibration analysis-based fault detection systems report enhanced accuracy when characterization of features in both time and frequency domains is performed simultaneously, which is accurately captured in this study. The strong role of RMS energy, spectral entropy, and peak FFT values immediately validates this result. Likewise, a study by Binder et al. (2023) demonstrated that a machine-based model incorporating input from multi-modal sensors provided a better early warning indication for faults compared to models of a threshold-level inspection system. A similar validation of algorithm-based methods providing increased reliability for fault detection has been obtained in this study.

Vibration and acoustic property trends in this analysis also confirm those of Le Nguyen (2023), who noted that defective rail sections display nonstationary vibration properties, larger kurtosis, and a peak in frequencies that next correspond to the motive. The visualization from the GitHub data set, using FFT techniques, replicated all these properties, from defective data points, with an abnormal harmonic peak and wide-band noise. This further ascertains that this data set has captured true trends of actual degradation in railway structures.

These findings also confirm theories of predictive maintenance, as defined in contemporary operations research literature. Zhang et al. (2021) highlighted, for instance, how the leverage of interpretable features, such as RMS energy and wavelet-based variability, would result in superior performance of predictive models. This report also indicates how feature interpretation by a random forest algorithm and PCA variance explanation imply that a model need not necessarily have either high interpretation potential or practical predictive utility. Again, the better performance of ensemble-based models, such as random forest, corresponds with recommendations in the literature on railway data monitoring, which often suggest that ensemble-based models are well-suited for sensor data.

Cumulatively, the machine learning results and behavior patterns identified in this study not only validate previous research but also validate and expand upon it. Utilizing an available GitHub data set enhances this validation, as it has been shown that a lack of publicly available data sets in this field has been a hindrance, as comparative studies are difficult with proprietary data sets. Similarly, this comprehensive model of acoustic and vibration characteristics presents a new perspective on the growing yet promising area of multi-modal sensing, which seeks to enhance early fault detection in railways.

## 6.3 Practical Implications for Railway Operations

The results obtained in this study have practical significance for contemporary railway systems, especially as railways progressively adopt digitized data management approaches. The superior prediction capabilities of models such as Random Forest, SVM, and XGBoost indicate that machine learning models have a robust potential for detecting defective rail sections using vibration and acoustic features derived from the GitHub Railway Track Fault Detection Dataset. This implies that railways can detect early signs of rail degradation far before conventional teams would develop diagnoses of similar defects. With Python, the automated early detection of faults in rail segments would no longer rely on manual walkthrough tests, which are vulnerable to missing early signs of defects.

Additionally, one of the most significant implications of this technology for train companies is the improved management of maintenance tasks. Since these models have been proven to be precise and consistent when cross-validated, it becomes easier for these teams to reliably predict hotspots where their trains are most likely to fail. This, in turn, reduces unplanned maintenance, minimizes network disruptions, and enhances overall network reliability. Additionally, since their predictions can be linked with other computerized maintenance management systems in use, there is no compulsion to maintain their equipment in a rigidly planned manner, relying solely upon a time-based mechanism, which reduces costs as per previous studies.

Additionally, insights from this research can also inform strategies aimed at enhancing safety. Defective track components contribute to derailments, speed restrictions, and passenger discomfort. The integration of automation for identifying defective vibration patterns, such as kurtosis values, ventricles, and FFT harmonics, provides a platform for early warning systems that can alert train operators to emerging hazardous conditions. These systems can even be incorporated into train diagnostic boxes and/or stations along the tracks for constant surveillance.

In addition, the results are consistent with the emergence of new technology in digital twinning, where actual sensor data in real-time conditions is used to create a simulation of railway infrastructure. The use of multi-modal vibration and acoustic features in this study makes it ideal for simulation in digital twinning, where precise modeling of mechanical properties under load is a priority. These models help create a safer rail transport environment by being proactive, aligning with trends in intelligent transport systems.

## 6.4 Theoretical Contributions

This research also advances the theoretical frontiers in several important ways, particularly in data-driven predictive maintenance and rail condition monitoring. To begin with, since this research utilized an open-source, multi-modal sensor data set, it has, in a direct manner, attempted to address a long-standing problem in the related literature, namely the lack of accessible, high-quality data sets for replication in model studies. Most existing model studies are tied to industrial data sets that are not open-source, which reduces their replicability and, by extension, their potential for use in scholarly studies.

The second most important theoretical contribution involves a model that combines features from both time-domain and frequency-domain analyses. While other studies have focused on vibration amplitude measures in their raw form, this research has demonstrated that an integrated approach combining RMS values, kurtosis, spectral entropies, and FFT features yields a significant improvement in classification performance. This supports theories that defend feature integration techniques in structure monitoring tasks, and it also builds upon previous successful experiments that utilized different modes of sensing to capture various characteristics of mechanical degradation.

A third, and in this area rather important, aspect of this work can be found in the model development pipeline, which is completely Python-based and provides a transparent model development process. Many other studies merely offer a conceptual overview of their models, without providing reproducible computational black boxes. This work, in marked contrast, contains completely reproducible computational code for every aspect, including but not limited to feature extraction, model development, performance analysis, visualization, and significance tests. This, in turn, ensures that this particular work, as well as other similar works in this area, can be easily replicated in industry and academia.

Lastly, this research bridges all identified research gaps in Chapter 2, encompassing the analysis of important features, sensor signal characteristics, and PCA-related dimensionality issues. These research findings contribute to a better understanding of the characteristics of vibration/acoustic wave patterns that are most indicative of defects, thereby complementing theoretical models of rail-track degradation patterns. Notably, the PCA analysis results, particularly in the scatter plots comparing healthy and defective cases, provide evidence to support theoretical proposals that minor structural defects can be identified in terms of wave characteristics during the early stages of degradation. This idea underpins the fundamental theories in data-driven techniques, which not only improve defect detection but also facilitate an understanding of the theoretical concepts underlying degradation patterns.

## 6.5 Limitations of the Study

The results of this capstone project should be considered in light of certain limitations in methodology and data. Firstly, it has been noted that though the open-source Railway Track Fault Detection Dataset was a good starting point for a model-building exercise, it has certain limitations in terms of representation. This data has been gathered using a specific method in a limited setup. Therefore, it would be difficult to assume that it has considered all factors, such as differences in axle pressures, variations in environmental conditions, differences in rail types, and degradation over time.

The second limitation relates to class imbalance in the labeled fault classes. There were relatively many regular segments in comparison to those that were cracked or misaligned. This model performance issue, despite cross-validation and scaling, suggests that the classification models encountered some difficulty in detecting important fault features, despite their low incidence in the dataset. This echoes a common issue reported in various studies on predictive maintenance, where data with faults, for instance, is relatively sparse.

Another limitation, which is imposed by the nature of signal processing, has been identified in this research. The classical approaches to feature design, including RMS, kurtosis, spectrum entropy, and FFT, were predominantly used in this work for extracting frequencies. Although these features are well-proven, their capabilities to capture the transient characteristics of rail-wheel interaction can be limited in some cases, due to the nonlinear nature of these phenomena.

Moreover, there were certain computational constraints in applying conventional machine models. Although these models are Explainable, models like Random Forest, SVM, and XGBoost could fare worse in comparison to Deep Learning models when it comes to processing raw wave patterns with high frequency. This analysis, carried out on processed feature sets rather than raw data streams, restricted its application to complex end-to-end models.

Finally, some minor methodological issues arose due to the use of open-source software tools and a Python-driven development workflow. Although it was very easy to trace and reproduce the analysis data flow, it should be noted that a lack of industrial-level calibration, a lab-controlled setting, and hardware-synchronized multi-sensor data input limited the accuracy of this work as an accurate representation of a production-level predictive maintenance setting.

## 6.6 Recommendations for Future Research

Further research in this area can benefit from the findings presented in this study by extending itself to investigate even more sophisticated modeling structures. This can be achieved by adopting a comprehensive data acquisition plan that incorporates a variety of sensors, including accelerometers, microphone sensors, wheel impact load sensors, temperature sensors, and laser-based geometry measurement systems. This would help in acquiring a different set of information related to the performance of the rail track.

Researchers should also explore deep-learning approaches, especially convolutional neural networks, long short-term memory networks, and combinations of both. Deep-learning models are ideally suited for raw waveforms and can identify complex patterns in waveforms within a model without relying on manually extracted features. With the current advancements in vibration-based structural diagnostics, deep models would greatly benefit and perform better than conventional models in machine learning.

A third area for further research would be leveraging concepts of transfer learning. This would be due to inhomogeneous rail networks with different geographic and functional capacities, as models would then lack generalizability when trained in a particular environment using a specific dataset. Frameworks that employ concepts of transfer learning make it feasible to use models for different open-source datasets in a new setting with relatively low data.

Additionally, in future studies, variables related to environmental conditions, such as weather and seasonal patterns, as well as train load characteristics, can be incorporated to achieve better results. These environmental variables typically influence the characteristics of rail vibration; therefore, analyzing them in conjunction with additional variables would facilitate a comprehensive understanding of this topic.

Lastly, it remains for researchers to incorporate further enhancements in statistical model evaluation methodologies, for example, multi-group ANOVA, in order to test the significance of their models with respect to various types of faults. This would enhance the analytical capabilities for determining model robustness in making predictions. With all these advancements, predictive maintenance technology will facilitate the development of intelligent, scalable, and automated decision support systems for railway infrastructure.

## 6.7 Conclusion

This research project aims to meet this increasing demand for efficient predictive models for predictive maintenance in railways, as it proposes and tests models inspired by machine learning using a sensor data set available for public use. To frame this research, this problem statement explains that, currently, railways are suffering from a lack of effective predictive models for predictive maintenance due to standard practices of preventative checks that are often ineffective in detecting early signs of degradation in railway structures.

Leaning on the Railway Track Fault Detection Dataset available on GitHub, the study has established a model that has been completely transparent, reproducible, and follows the latest trends in condition monitoring. The Python-based analysis, which involves data preprocessing, feature development, supervised learning, validation of models, and statistical significance tests, has established models with high predictive capabilities. The Random Forest Classifier and XGBoost models in this study were able to verify the hypotheses as well as the models established in previous chapters.

Indeed, it has been found that sensor-based features like RMS vibration energy, spectral entropy, kurtosis, and FFT magnitude are of great diagnostic use. These were in line with various findings in the latest scholarly studies, and the outcome from signal analysis showed characteristic patterns for healthy vs. faulty classes, which were indicative of abnormal structures in their respective harmonic peaks and energy distribution. This further manifests that multimodal sensors are able to identify minute structural defects, which are usually missed by conventional analysis.

From a practical point of view, this study points out the benefits of predictive maintenance in terms of efficient scheduling of inspections, a decrease in downtimes, and a further improvement in rail network performance in terms of safety. Again, this study proves that machine learning integration in digital rail technology is gaining importance.

Despite this, it appears that there are a number of limitations that are acknowledged in this study, which are related to the representation of data, potential class imbalance, as well as traditional machine learning model constraints. These are in themselves opportunities for further studies, including investigating deep models, other sensor channels, and model validation on different rail scenarios.

In sum, it can be seen that this research offers a compelling example of how open-source sensor data and a Python-based machine learning approach offer a marked improvement in railway fault detection. It not only bridges theoretical understanding with practical application but also adds to the growing momentum that railways must move towards a data-driven, safety-oriented approach as part of a predictive maintenance model.

# REFERENCES

Aremu, O. (2020). Achieving a representation of asset data conducive to machine learning driven predictive maintenance. University of Queensland. https://espace.library.uq.edu.au/view/UQ:c203a9b

Binder, M., Mezhuyev, V., & Tschandl, M. (2023). Predictive maintenance for the railway domain: A systematic literature review. IEEE Engineering Management Review, 51(2), 120–140. https://ieeexplore.ieee.org/abstract/document/10082880/

Chin, C. H. (2024). A review of the wavelet transform for durability and structural health monitoring. Engineering Failure Analysis (review article). https://www.sciencedirect.com/science/article/pii/S1110016824004514

Ejaz, U., & Bakhsh, M. M. (2023). AI-driven predictive maintenance in autonomous public transit systems for smart cities. World Journal of Advanced Research and Reviews, 17(1), 1400–1415. https://doi.org/10.30574/wjarr.2023.17.1.0129

GitHub Repository (dataset): durgeshsahu16. (n.d.). Railway-Track-fault-Detection-Project. Retrieved December 8, 2025, from https://github.com/durgeshsahu16/Railway-Track-fault-Detection-Project

Inoue, K. (2024). Fault diagnosis of railway assets using acoustic signals (PhD thesis). University of Birmingham. https://etheses.bham.ac.uk/id/eprint/14695/7/Inoue2024PhD.pdf

Le Nguyen, M. H. (2023). Online machine learning-based predictive maintenance for the railway industry (Doctoral dissertation). https://theses.hal.science/tel-04164338/

Lee, J. H., Kim, H., & Park, S. (2023). A study on wheel member condition recognition using vibration signals and machine learning. Sensors, 23(20), 8455. https://doi.org/10.3390/s23208455

Łuczak, D. (2024). Machine fault diagnosis through vibration analysis: Time-, frequency- and time-frequency domain methods. Electronics, 13(2), 452. https://doi.org/10.3390/electronics13020452

Minguell, M. G., et al. (2023). TrackSafe: A comparative study of data-driven techniques for railway track fault detection. Engineering Applications of Artificial Intelligence, 120, 105185. https://doi.org/10.1016/j.engappai.2023.106622

Ojeda, J. C. O., et al. (2025). Application of a predictive model to reduce unplanned downtime: A sustainability perspective. Sustainability, 17(9), 3926. https://doi.org/10.3390/su17093926

Prakash, A., & Tadepalli, T. (2024). A review of acoustic signal-based detection of damage in railway tracks. In Proceedings of IEEE Conference on Engineering Informatics (ICEI 2024). https://ieeexplore.ieee.org/document/10912263

Ravi Aravind, C. V. S., & Surabhi, M. D. (2022). Machine learning applications in predictive maintenance for vehicles: Case studies. International Journal of Engineering and Computer Science, 11(11), 25628–25640. https://doi.org/10.18535/ijecs/v11i11.4707

Shafique, R., Siddiqui, H. U. R., Rustam, F., Ullah, S., & Lee, J. (2021). A novel approach to railway track faults detection using acoustic analysis. Sensors, 21(18), 6221. https://doi.org/10.3390/s21186221

Siddiqui, H. U. R., Saleem, A. A., Raza, M. A., Zafar, K., Munir, K., & Dudley, S. (2022). IoT-based railway track faults detection and localisation using acoustic analysis. IEEE Access. https://doi.org/10.1109/ACCESS.2022.3210326

Wróbel, J., et al. (2024). Fault detection and diagnostic methods for railway systems: A review. Automation in Construction, 150, 104803. https://doi.org/10.12913/22998624/191762

Xie, J., Huang, J., Zeng, C., Jiang, S.-H., & Podlich, N. (2020). Systematic literature review on data-driven models for predictive maintenance of railway track: Implications in geotechnical engineering. Geosciences, 10(11), 425. https://doi.org/10.3390/geosciences10110425